MaintainIQ: An Explainable MongoDB-Native Decision Support Platform for Industrial Asset Health Monitoring

Divya (23MIA1099)  , LokiReddy Madhavi (23MIA1135)  |  Prassana J | TARP |  Vellore Institute of Technology, Chennai

| Motivation / Introduction. Industrial systems rely on continuous sensor measurements to avoid unexpected machine failure. While anomaly detection identifies abnormal behaviour, engineers also need to know how serious the condition is, what contributed to the alert, and which asset should be inspected first. MaintainIQ connects sensor analytics to maintenance decisions through interpretable health scoring, prioritization and rule-based explanations.. Scope and Objectives. • Ingest MetroPT-3 APU sensor data into MongoDB time-series collections. • Learn normal temporal patterns with an LSTM Autoencoder and flag deviations. • Compute a Dynamic Asset Health Index (DAHI) from sensor-level scores. • Rank maintenance attention with MAPE using health, risk and criticality. • Provide EME inspection recommendations and AMA natural-language queries.. Methodology — End-to-End Processing Flow. Eight connected stages transform raw observations into an engineer-facing decision. Fig. 1. Connected MaintainIQ methodology: sensing → analysis → maintenance decisions MongoDB stores and aggregates time-series readings; Python/TensorFlow performs LSTM inference. The DAHI score is derived from sensor deviation, while MAPE and EME turn analytical outputs into maintenance priority and explanations.. Implementation Details. Stage; Method / Configuration. Input; MetroPT-3: 1,516,948 records; 15 channels; four documented failures. Preprocessing; 10-second → 1-minute; mean/mode; 5-point smoothing; Z-score. LSTM model; 60-step × 10-feature inputs; encoder 64/32; latent 16; decoder 32/64. Anomaly rule; Reconstruction MSE above 95th-percentile training threshold. Health scoring; MongoDB aggregation combines weighted sensor scores. Decision layer; MAPE priority, EME rules, AMA approved query intents.. Methodology Visuals and Results. Fig. 2. Model training loss; Fig. 3. Anomaly scores over time. Fig. 4. COMP_01 DAHI gauge; Fig. 5. Sensor health scores. Figs. 2–3: Training MSE declines, and reconstruction-error peaks cross the chosen threshold. Figs. 4–5: COMP_01 has low asset health, with H1 and oil temperature contributing the weakest scores.. Implementation Inference. The project does more than label a record as normal or anomalous. The connected modules translate sensor evidence into an interpretable health assessment, a maintenance queue and suggested follow-up checks. The explanation identifies plausible causes for review, not a confirmed physical diagnosis.. Related-Work Comparison. Approach; Contribution; Remaining gap. Alhammadi et al.; Autoencoder APU anomaly detection; No ranked maintenance decisions. XAI methods; Explanations of model predictions; Limited direct maintenance guidance. Industrial NoSQL; Flexible sensor-data management; Often used mainly for storage. MaintainIQ; Detection + health + ranking + explanation; Field-scale validation remains future work.. Operational Interpretation. When a compressed-air system shows abnormal pressure and load patterns, an anomaly score triggers closer analysis rather than an automatic repair order. DAHI exposes weakened sensor contributions; MAPE ranks the asset for attention; EME supplies a possible inspection path for review by a qualified engineer.. | Results — Anomaly Detection. The LSTM Autoencoder was evaluated using reconstruction-error detection. The reported best model results are shown below; the base paper’s earlier MLP autoencoder is distinct from the LSTM used in MaintainIQ. Measure; Value; Interpretation. Best F1-score; 0.9869; Strong precision–recall balance in the best evaluated setting. Maximum ROC-AUC; 0.999; High discrimination across score thresholds. Train MSE; 0.0168; Final training reconstruction loss. Validation MSE; 0.0349; Held-out reconstruction loss. Threshold; 95th pct.; Training-error percentile used to flag anomalies. Inference: the model distinguishes many anomalous sequences from learned normal behaviour, but performance varies by failure split and operating period.. Results — Model and Database Visuals. Fig. 6. Signal attention visualization; Fig. 7. Database comparison (%). Fig. 8. Sensor health radar; Fig. 9. Comparative workload chart. Inference: the health plots expose uneven sensor conditions; the benchmark comparison suggests benefits for MongoDB under the tested time-series aggregation workload, not for every possible database workload.. Evaluation Metrics — Asset Decision Output. Output; Observed; Meaning. DAHI: COMP_01; 39.66 / 100; Critical / near-critical asset condition; requires attention. H1 health; 39.4 / 100; Weak sensor-health contributor. Oil-temperature health; 49.9 / 100; Additional weakened contributor. MAPE priority; 84.1; High-ranked maintenance attention. EME; Rule based; Readable inspection recommendations. AMA; 5 intents; Retrieve health, risk, history and sensor readings. End-to-end case: abnormal temporal behaviour → COMP_01 health 39.66 → MAPE 84.1 → review H1 and oil-temperature evidence → inspect according to approved maintenance procedures.. MongoDB vs MySQL — Benchmark Findings. Workload indicator; MongoDB relative finding; Meaning. Storage footprint; 31.0% smaller; Compact tested data storage. Time-window query; 43.8% lower cost; Faster selected window analysis. Aggregation query; 72.2% lower latency; Faster tested aggregation. Schema modification; 99.9% less time; Flexible new-field adaptation. These differences concern one benchmark setup. Hardware, indexes, query definitions and repeated timing logs are needed before generalizing beyond the measured workload.. Conclusion / Summary. MaintainIQ integrates a temporal anomaly detector with MongoDB-native health analytics and maintenance decision support. It turns raw anomaly flags into graded asset health, ranked urgency, rule-based explanations and natural-language access. The main contribution is the unified workflow, rather than a new standalone algorithm. The current evaluation uses MetroPT-3 from one APU with four documented failure periods. Live operational benefit, reduced downtime and large-scale generalization still require field validation.. Acknowledgement / References. Acknowledgement: Vellore Institute of Technology, Chennai, and the researchers who made the MetroPT dataset and related work available for academic study. [1] M. Alhammadi, I. Zaiter and R. Jaradat, “An Autoencoder-Based Framework for Time-Series Anomaly Detection in Metro Train Air Production Units,” IEEE GCAIoT, 2025. [2] B. Veloso et al., “The MetroPT dataset for predictive maintenance,” Scientific Data, 2022. [3] N. Davari et al., “A survey on data-driven predictive maintenance for the railway industry,” Sensors, 2021. [4] L. Cummins et al., “Explainable predictive maintenance: A survey,” IEEE Access, 2024.. Explainable Maintenance Rules — Examples. Observed condition; Interpretation / inspection cue. Pressure drop + rising current; Possible leakage or load issue; examine pneumatic path. High oil temperature; Inspect lubrication level and cooling. Low reservoir pressure + LPS flag; Review insufficient air-production condition. These are transparent engineering heuristics, not validated causal diagnoses. The maintenance team makes the final operational decision.. Next Steps / Future Scope. Evaluate on additional APUs and operating environments; audit alert burden and false alarms; test the explainability rules with maintenance engineers; validate database benchmarks under controlled hardware settings; and assess real deployment benefits such as faster fault triage.. |
| --- | --- |